SIG SOUTHERN INDUSTRIAL GAS SDN BHD

SAFETY DATA SHEET

HYDROGEN SULFIDE AND NITROGEN BALANCE

- CHEMICAL PRODUCT AND COMPANY IDENTIFICATION

| Product name | Hydrogen Sulfide 20ppm Balance Nitrogen |
| --- | --- |
| Synonyms | - |
| Chemical Formula | H2S (Hydrogen Sulfide) , N2 (Nitrogen) |
| CAS No | 7783-06-4 (Hydrogen Sulfide) ; 7727-37-9 (Nitrogen) |
| Use of Substance | Environmental calibration gas |
| Manufacturer | SOUTHERN INDUSTRIAL GAS SDN. BHD. |
|  | PLO 137, Kawasan Perindustrian Senai III, |
|  | 81400 Senai, Johor. |
| Contact Number | 07-598 3863 |
| Emergency Phone Number (24 hr) | 07-598 3863 |

- HAZARDS IDENTIFICATION

| Chemical Name | CAS No. | Classification Code | Labeling |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | H-code | Signal Word | Hazard Pictogram |
| Hydrogen Sulfide 20ppm Balance Nitrogen | 7783-06-4 (Hydrogen Sulfide); 7727-37-9 (Nitrogen) | Press. Gas | H 280 | Warning |  |

| Classification of the substance | Press. Gas | : | Gases under pressure (Compressed gas) |
| --- | --- | --- | --- |
| Hazard Statement | H 280 | : | Contains gas under pressure; may explode if heated. |
| Precautionary Statement | P403 | : | Store in a well-ventilated place. |
| Other Hazards | Irritation to the eyes, mucous membranes, and upper respiratory tract. Exposure to Hydrogen Sulfide for more than 30 minutes at concentration of greater than 600 ppm have been fatal. Continuous inhalation of low concentrations may cause olfactory fatigue, so that the odor is no longer an effective warning of the presence of Hydrogen Sulfide. Mixture acts as a simple asphyxiant by displacing air necessary for life. Symptoms include rapid respiration, muscular incoordination, fatigue, dizziness, nausea, vomiting, unconsciousness, and death. |  |  |

- COMPOSITION/INFORMATION ON INGREDIENTS

| Common Name | Ingredient | CAS Number | % volume | OSHA-PEL |
| --- | --- | --- | --- | --- |
| Hydrogen Sulfide 20ppm Balance Nitrogen | Nitrogen | 7727-37-9 | >99.9 | None established |
|  | Hydrogen Sulfide | 7783-06-4 | ≤ 0.001 - 0.1 | 10 ppm |

*Contains no other components or impurities which influence the classification of the product.

- FIRST AID MEASURES

| Eye Contact |  |  | Flush eyes with plenty of water for at least 15 minutes. Seek immediate medical attention |
| --- | --- | --- | --- |
| Inhalation |  |  | Immediately remove victim to fresh air. If breathing stopped, give artificial respiration. If breathing is difficult, give oxygen. Get immediate medical attention. |
| Skin Contact |  |  | Wash with water for at least 15 minutes while removing contaminated clothing. Seek immediate medical attention |
| Ingestion |  |  | Seek immediate medical attention |
| Most important symptoms and effects, both acute and delayed |  |  | Exposure to Hydrogen Sulfide for more than 30 minutes at concentration of greater than 600 ppm have been fatal. Continuous inhalation of low concentrations may cause olfactory fatigue, so that the odor is no longer an effective warning of the presence of Hydrogen Sulfide. Mixture acts as a simple asphyxiant by displacing air necessary for life. Symptoms include rapid respiration, muscular incoordination, fatigue, dizziness, nausea, vomiting, unconsciousness, and death. |

- FIRE FIGHTING MEASURES

| Suitable extinguishing media |  |  | Carbon dioxide, regular dry chemical. |
| --- | --- | --- | --- |
| Unsuitable extinguishing media |  |  | None known |
| Special hazards arising from the chemical |  |  | Non flammable. Container may rupture or explode if exposed to heat. Hazardous combustion products : Sulfur oxides. |
| Special protective equipment and precautions for fire fighters |  |  | Cool containers with water spray until well after fire is out. Stay away from ends of tanks. Stop flow of gas. Use Self-contained breathing apparatus while in confined space. |

- ACCIDENTAL RELEASE MEASURES

| Personal precautions |  |  | Evacuate area. Provide maximum explosion-proof ventilation. Eliminate ignition sources. Post warning notices (including no smoking). |
| --- | --- | --- | --- |
| Environmental precautions |  |  | Try to stop release. Prevent from entering sewers, basements and work pits, or any place where its accumulation can be dangerous. Hydrogen is unlikely to cause an environmental hazard; however emergency responders should be aware of other substances that may be involved in the release. |
| Clean up methods |  |  | Provide adequate ventilation. |

- HANDLING AND STORAGE

| Precaution for safe handling |  |  | Operators should wear protective clothing while handling this gas. If ventilation controls are not adequate to provide sufficient oxygen content, proper respiratory protection equipment should be provided. |
| --- | --- | --- | --- |
| Condition for safe storage |  |  | Cylinders should be stored upright and be secured firmly to prevent falling. Protect cylinders against extreme weather and from dampness from ground to prevent rusting. Stored cylinders in well-ventilated area, away from direct heat and ignition source. Do not allow area where cylinders area stored to exceed 52oC. |

- EXPOSURE CONTROLS/PERSONAL PROTECTION

| Control parameters |  |  | INGREDIENT; Exposure Limit in Air. ACGIH-TLV; OSHA - STEL. TWA ppm; STEL ppm; TWA ppm; STEL ppm. Nitrogen Formula: N2; No specific exposure limits for Nitrogen. Hydrogen Sulfide Formula: H2S; 1; 5; 10 (Vacated); 15 (Vacated). |
| --- | --- | --- | --- |
| Appropriate engineering controls |  |  | Provide adequate general and local exhaust ventilation to maintain concentration below exposure limits and to avoid asphyxiation. Provide local exhaust ventilation system. Ensure compliance with applicable exposure limit. |
| Personal protection equipment |  |  | Eye protection recommended. Provide emergency eye wash fountain and quick drench shower in immediate work area. Protective industrial work gloves made of any suitable material. Under conditions of frequent use or exposure, respiratory protection may be needed. Wear safety shoes |

- PHYSICAL AND CHEMICAL PROPERTIES

| Appearance |  |  | Colorless, Gas |
| --- | --- | --- | --- |
| Odour |  |  | Rotten egg odor, colorless gas |
| Odour threshold |  |  | Not Applicable |
| pH |  |  | Not Available |
|  |  |  | The following information is for inert component (N2) |
| Melting point / Freezing point |  |  | -210 oC |
| Boiling point |  |  | -196 oC |
| Flash point |  |  | Not Available |
| Evaporation rate |  |  | Not Available |
| Flammability |  |  | Non flammable (nitrogen) Flammable (hydrogen sulfide) |
| Upper/lower explosive limit |  |  | For (hydrogen sulfide) LOWER: 4.0% UPPER: 44.0 % |
| Vapour pressure |  |  | Above Critical Temperature |
| Vapour density (Air =1) |  |  | 0.97 |
| Relative density |  |  | Not Available |
| Solubility (H2O) |  |  | 0.023 |
| Partition coefficient |  |  | Not Available |
| Auto ignition temperature |  |  | Not Available |
| Decomposition temperature |  |  | Not Available |
| Viscosity |  |  | Not Available |

- STABILITY AND REACTIVITY

| Reactivity |  |  | Unreactive under normal conditions. |
| --- | --- | --- | --- |
| Chemical Stability |  |  | Stable at standard temperatures within shelf-life |
| Possibility of hazardous reactions |  |  | Under normal conditions of storage and use, hazardous reactions will not occur. |
| Condition to avoid |  |  | Cylinders exposed to high temperatures or direct flame can rupture or burst. |
| Incompatible materials |  |  | Strong oxidizing agents. Bases. Metal Oxides |
| Hazardous decomposition products |  |  | Hydrogen gas. Sulfur oxides. |

- TOXICOLOGICAL INFORMATION

| Information on toxicological effects |  |  |  |
| --- | --- | --- | --- |
| This gas mixture is not expected to cause any Mutagenicity, Embryotoxicity, Teratogenicity, and Reproductive Toxicity. Animal reproductive data are available for Hydrogen Sulfide (Component of this gas mixture). |  |  |  |
| Acute toxicity |  |  | Oral: LD50 > No information available. |
|  |  |  | Dermal: LD50 > No information available. Inhalation: LC50 > No information available. |
| Skin corrosion / irritation |  |  | No specific data. |
| Serious eye damage/ irritation |  |  | No specific data. |
| Respiratory or skin sensitisation |  |  | No specific data. |
| Germ cell mutagenicity |  |  | No specific data. |
| Carcinogenicity product |  |  | No specific data. |
| Reproductive toxicity product |  |  | No specific data. |
| Specific target organ toxicity – single exposure product. |  |  | No specific data. |
| Specific target organ toxicity – repeated exposure product |  |  | No specific data. |
| Aspiration hazard product |  |  | Not applicable to gases and gas mixtures. |

- ECOLOGICAL INFORMATION

| Ecotoxicity effect |  |  |  |
| --- | --- | --- | --- |
| Acute toxicity product |  |  | This gas may have adverse effects on animal life exposed to very high concentrations. |
| Additional ecological information |  |  | This gas may have adverse effects on aquatic life. The following data are available for Hydrogen Sulfide, a component of this gas mixture. Fish Toxicity: Hydrogen Sulfide: 96 Hours 4.48 ug/L LC50, Lepomin macrochirus Invertebrate Toxicity: Hydrogen Sulfide: 22 ug/L 96 hours LC50, Gammarus psedolimnaeus |
| Persistence and degradability |  |  | Nitrogen is a neutral element and presents no hazard of persistence. Hydrogen Sulfide will convert to elemental sulfur upon standing in water. |
| Bioaccumulative potential |  |  | The components of this gas mixture do not have bioaccumulation or food chain contamination potential. |
| Mobility in soil |  |  | The components of this gas mixture will not be mobile in soil |
| Other adverse effects |  |  | No other adverse effects are identified |

- DISPOSAL CONSIDERATIONS

| Waste from residue / unused product |  |  | Do not attempt to dispose of residual waste or unused quantities. |
| --- | --- | --- | --- |
| Contaminated packaging |  |  | Return in the shipping container PROPERLY LABELED, WITH ANY VALVE OUTLET PLUGS SECURED AND VALVE PROTECTION CAP IN PLACE to an authorized distributor for proper disposal. |

- TRANSPORT INFORMATION

| UN Number |  |  | UN 1956 |
| --- | --- | --- | --- |
| UN proper shipping name |  |  | Compressed gas, n.o.s (Oxygen,Argon) |
| Transport hazard class(es) |  |  | 2.2 (Nonflammable) |
| Packing group |  |  | - |
| Environmental hazards |  |  | No |
| Special precautions for user |  |  | No |
| Transport in bulk according to Annex II of MARPOL73/78 and the IBC Code |  |  | Not applicable |
| Information |  |  | Ensure the driver is understand well on the potential hazards of the load and knows what to do in the event of an accident or an emergency. Secured the product containers before transporting it. Ensure that the cylinder valve is closed and not leaking. Container valve guards or caps should be in place. Ensure adequate air ventilation. |

- REGULATORY INFORMATION

Contact local government authority.

- OTHER INFORMATION

| Date of Preparation / Revision of SDS |  |  | 9-September-2014 / Rev. 00 |
| --- | --- | --- | --- |
| Legend to the abbreviations ad acronyms used |  |  |  |
| Classification of the substance | Press. Gas | : | Gases under pressure (Compressed gas) |
| Hazard Statement | H 220 | : | Extremely flammable gas |
|  | H 280 | : | Contains gas under pressure; may explode if heated. |
| Precautionary Statement | P403 | : | Store in a well-ventilated place |
| Abbreviations | LC50 | : | median lethal concentration |
|  | LD50 | : | median lethal dose |
|  | PEL | : | Permissible exposure limits |

Although reasonable care has been taken in the preparation of this document we extend no warranties and make no representations as to the accuracy or completeness of the information contain herein, and assume no responsibility regarding the suitability of this information for the user’s intended purposes or for the consequences of its use. Each individual should make a determination as to the suitability of the information for their particular purpose(s)